ECA5604  Advanced Wireless Communication Systems

CO1 Integrated Engineering Analysis and Planning Assignment

Evolution, cellular planning, spectrum use and interference management

| Assessment format | Duration | Maximum marks | Bloom levels |
| --- | --- | --- | --- |
| Open-ended engineering design report | 1 week | 100 | K4 Analyze · K5 Evaluate · K6 Create |

Student Details

Student Name: J.Dharshini

Register Number: 1912260392

Department / Year: ECE/First

Submission Date: 28.10.2026

Course outcome assessed

CO1: Understand and articulate the evolution of mobile communication systems, cellular architecture, spectrum utilization, and interference management in wireless communication networks.

Engineering design problem

A regional wireless operator is planning the next stage of its mobile network. Using the planning data provided, prepare an evidence-based engineering report that explains the evolution of mobile systems from 1G through the 6G vision, and how cellular layout, traffic demand, mobility, spectrum and interference shape deployment decisions. Prepare one evidence-based engineering report for the planning team. Use the supplied data to analyze the linked design tasks, explain the underlying concepts, state assumptions, and defend a coherent recommendation. There is no single predetermined network choice; marks depend on correct analysis and technically sound reasoning.

Planning data

| Topic | Supplied data |
| --- | --- |
| Cell geometry | Hexagonal model; cell radius r = 2 km; planning cluster = 19 cells; reuse cluster size N = 7. |
| Traffic and channels | Planning area demand = 1,000 subscribers at 0.08 Erlang each in the busy hour; an initial base station has C = 100 traffic channels; target blocking probability ≤ 2%. |
| Mobility | Mobile speed = 60 km/h; overlap-zone dwell time = 6 s; nominal cell length along the route = 1 km; handoff interruption = 0.5 s; target cell unavailable 1% of the time. |
| Interference case | Desired signal S = −70 dBm; co-channel interference I₁ = −85 dBm; adjacent-channel interference I₂ = −90 dBm; minimum target SINR = 15 dB. |
| Spectrum comparison | Compare 900 MHz, 3.5 GHz and 28 GHz at distance d = 100 m. Use c = 3 × 10⁸ m/s and FSPL(dB) = 20 log₁₀(4πd/λ). |
| Generational examples | 1G AMPS: 800 MHz, 30 kHz per analog FDMA channel, 10 MHz total allocation. 2G GSM: 900 MHz carrier, 200 kHz carrier bandwidth, 8 TDMA slots. 3G UMTS: 5 MHz carrier. 4G LTE: OFDMA and MIMO. 5G NR: eMBB/URLLC/massive IoT, mid-band and mmWave, beamforming. 6G vision: THz, AI-native networking, holographic communication, tactile internet and ISAC. |

Use Erlang B for the blocking calculation. You may use a spreadsheet or a short script for numerical evaluation, but show the model, inputs and interpretation. For the idealized 1G channel count, state whether guard bands and duplexing are ignored.

Stage 1  Explain the evolution of mobile systems

Trace the progression from 1G through the 6G vision using the examples in the planning data. For each generation, identify its main service capability, access or radio technology where specified, and one important limitation or engineering change that motivated later generations. Explain how this progression changed the role of spectrum, capacity and interference management.

Calculate the wavelength at 800 MHz and the idealized number of 30 kHz channels in 10 MHz. State the practical qualification to that idealized result. Relate the 2G move to digital transmission and TDMA to an improvement over 1G analog service.

Deliverable: a concise generational comparison table and a short synthesis paragraph.

Stage 2  Analyze cell coverage and frequency reuse

Using the hexagonal planning model, calculate the area of one cell and the total area of the 19-cell cluster. For N = 7, calculate the frequency reuse fraction and the reuse distance D = r√(3N). Explain the assumptions behind the hexagonal approximation and distinguish geometric coverage from guaranteed radio coverage.

Explain how cell splitting, sectoring, microcells, picocells and femtocells can change coverage and capacity. Recommend a suitable capacity-expansion measure for the regional operator and identify its effects on interference and handoff frequency.

Stage 3  Estimate traffic capacity

Calculate the offered busy-hour traffic for the planning area. Find the Erlang B blocking probability with 100 channels, then determine the smallest channel count that meets the 2% target. Estimate how many 100-channel base-station channel groups are needed to serve that channel requirement, and state why this is only a first-order estimate rather than a complete site plan.

Explain the capacity, blocking and infrastructure trade-offs in your result. State at least one assumption behind the offered-traffic model.

Stage 4  Evaluate mobility and handoff

Calculate the distance travelled during the 6 s overlap interval. Estimate hard handoffs per minute using the 1 km route cell length and supplied speed, stating how you interpret that length. Compare hard, soft and softer handoff in terms of connection continuity and resource use. Using the given interruption duration and 1% target-cell unavailability, estimate a simple call-drop risk for one handoff and explain the limits of that estimate.

Stage 5  Quantify interference and compare spectrum bands

For the interference case, calculate the carrier-to-interference ratio using the co-channel interferer alone. Then sum co-channel and adjacent-channel interference in linear power units, convert the total back to dBm, calculate SINR, and decide whether the 15 dB target is met. Recommend one suitable mitigation and explain its likely trade-off.

For 900 MHz, 3.5 GHz and 28 GHz, calculate wavelength and free-space path loss at 100 m, including path-loss differences relative to 900 MHz. Compare likely penetration, antenna size and coverage/capacity implications. Explain why the higher path loss at higher frequencies does not by itself determine achievable network performance.

Stage 6  Explain spectrum coordination and make a deployment recommendation

Describe the distinct roles of ITU-R, the World Radiocommunication Conference (WRC) and the Radiocommunication Bureau (BR) in international spectrum coordination. Explain one way international coordination affects national regulators and operators.

Conclude with a recommended staged strategy for the regional network. Your recommendation must connect generational evolution, the calculated cell and traffic plan, handoff behavior, interference results, and spectrum-band characteristics. Identify the most important assumptions or uncertainties that should be checked before deployment.

Submission requirements

Submit one engineering design report. Organize it as: requirements → analysis → calculations → comparison → recommendation. Include equations and units, clearly labeled tables or diagrams where useful, assumptions, and brief interpretations alongside numerical results. Cite any external technical sources used. Software output without a description of the method and engineering meaning is insufficient.

Assessment rubric

| Assessment criterion | Marks | Primary Bloom level |
| --- | --- | --- |
| Generational evolution and interpretation of CO1 concepts | 15 | K4 |
| Cell geometry, reuse and capacity-expansion analysis | 15 | K4 / K5 |
| Traffic and channel-capacity calculation | 15 | K4 / K5 |
| Mobility and handoff analysis | 15 | K4 / K5 |
| Interference calculations and mitigation reasoning | 20 | K4 / K5 |
| Spectrum-band comparison, ITU coordination and integrated recommendation | 20 | K5 / K6 |
| TOTAL | 100 |  |

Performance expectations

| Excellent | Good |  | Satisfactory | Needs improvement |
| --- | --- | --- | --- | --- |
| Synthesizes CO1 concepts, calculates accurately, states assumptions, and defends an integrated deployment recommendation with evidence. | Uses sound methods and gives a supported recommendation, with minor calculation or explanation gaps. | Completes the main calculations but gives limited interpretation or weak connections among topics. |  | Relies on unsupported statements or tool output; major concepts, calculations or engineering reasoning are missing. |

Consolidated Question

A regional wireless operator is planning the next stage of its mobile network. Using the planning data provided, prepare an evidence-based engineering report that explains the evolution of mobile systems from 1G through the 6G vision; analyzes cell coverage, frequency reuse, traffic demand, channel capacity, mobility, and handoff performance; calculates and interprets the effects of co-channel and adjacent-channel interference; compares the propagation and deployment implications of 900 MHz, 3.5 GHz, and 28 GHz; and explains how international spectrum coordination affects deployment. Conclude with a justified, phased network plan that connects your calculations and analysis, identifies relevant trade-offs, and states the assumptions and uncertainties that should be checked before implementation.

EXECUTIVE SUMMARY

This report evaluates a regional mobile network using the planning values supplied in the assignment. It reviews the evolution from 1G to the 6G vision, estimates hexagonal cell coverage and frequency reuse, evaluates busy-hour traffic using Erlang B, examines mobility and handoff behavior, quantifies interference, compares propagation at 900 MHz, 3.5 GHz and 28 GHz, and summarizes international spectrum coordination.

The calculations indicate a theoretical hexagonal cell area of 10.392 km², a 19-cell geometric area of 197.454 km², and a reuse distance of 9.165 km for N = 7. Offered traffic is 80 Erlangs. Under the Erlang B assumptions, 100 channels give approximately 0.399% blocking, while 92 channels are the minimum to meet the 2% target.

The supplied interference case gives a 15 dB co-channel-only C/I, but approximately 13.807 dB when the adjacent-channel interferer is included and thermal noise is neglected.

The recommended plan is to use lower-frequency spectrum for broad coverage, add mid-band capacity where traffic requires it, and use 28 GHz selectively for dense hotspots after measurements and interference coordination.

1. REQUIREMENTS AND PLANNING INPUTS

The objective is to recommend a technically justified, phased deployment strategy for a regional wireless operator. The report uses the planning data in the assignment and states assumptions where the data do not fully specify a model.

2. STAGE 1 – EVOLUTION OF MOBILE COMMUNICATION SYSTEMS (15 MARKS)

2.1 Generational Comparison

| Generation | Radio/Access Technology and Example | Main Capability | Limitation / Reason for Later Evolution |
| --- | --- | --- | --- |
| 1G | AMPS; analog FDMA; 800 MHz; 30 kHz per channel | Analog mobile voice | Limited capacity and analog privacy/quality limitations encouraged digital systems. |
| 2G | GSM; 900 MHz; 200 kHz carrier; 8 TDMA slots | Digital voice and SMS | Data capability was limited compared with later mobile broadband systems. |
| 3G | UMTS; 5 MHz carrier | Mobile data and multimedia services | Increasing data demand encouraged higher-capacity broadband radio systems. |
| 4G | LTE; OFDMA and MIMO | High-speed mobile broadband | New use cases increased the need for flexible latency, capacity and massive device connectivity. |
| 5G | NR; mid-band/mmWave; beamforming | eMBB, URLLC and massive IoT | Coverage and performance vary with band, propagation, deployment density and device capability. |
| 6G Vision | THz research, AI-native networking, ISAC | Potential holographic communication and tactile internet applications | These are forward-looking goals; propagation, hardware, energy and deployment feasibility require validation. |

Interpretation

Across generations, the network has moved from voice-focused analog radio to digital services, broadband data and more diverse service requirements. Spectrum management must balance coverage, capacity, service quality and interference.

Higher frequencies can provide wider bandwidth, but generally require more careful coverage planning.

2.2 Wavelength at 800 MHz

Given:

Speed of light, c = 3 × 10⁸ m/s
Frequency, f = 800 MHz = 800 × 10⁶ Hz

Formula:

λ = c / f

Substitution:

λ = (3 × 10⁸) / (800 × 10⁶)

λ = 0.375 m

Therefore,

Wavelength = 0.375 m = 37.5 cm

Result and Interpretation

The wavelength at 800 MHz is 0.375 m. This is a physical radio-wave wavelength, not a measure of cell radius or data rate.

2.3 Idealized Number of 30 kHz Channels in 10 MHz

Given:

Total bandwidth = 10 MHz
Individual channel bandwidth = 30 kHz

Convert the allocation:

10 MHz = 10,000 kHz

Formula:

Idealized channel count = Total bandwidth / Bandwidth per channel

Calculation:

= 10,000 / 30

= 333.33

Therefore, 333 complete 30 kHz channels fit in the idealized calculation, using 9,990 kHz and leaving 10 kHz unused.

Practical Qualification

This count ignores guard bands and duplexing, as the assignment requests. A real channel plan may have fewer usable channels because spectrum may be needed for transmit/receive separation, guard bands and regulatory or system constraints.

2.4 Why Digital 2G Improved on Analog 1G

1G analog FDMA allocates separate frequency channels to users. In the supplied GSM example, 2G uses digital transmission and divides a 200 kHz carrier into eight TDMA time slots.

Digital transmission enabled services such as SMS and supported more flexible radio-resource management.

The actual number of simultaneous users depends on the system design, control channels, coding, frequency reuse and other overheads. Eight slots should not be interpreted as eight guaranteed users in every operational circumstance.

Stage 1 Synthesis

Mobile systems evolved from analog voice to digital voice and messaging, multimedia, broadband, and networks designed for enhanced mobile broadband, reliable low-latency services and massive IoT.

6G remains a vision under development. Throughout this evolution, efficient spectrum use, adequate capacity, coverage and interference control have become central design requirements.

3. STAGE 2 – CELL COVERAGE AND FREQUENCY REUSE (15 MARKS)

3.1 Area of One Hexagonal Cell

For a regular hexagon whose radius r is the distance from its center to a vertex:

Formula:

A = (3√3 / 2)r²

Given:

r = 2 km

Calculation:

A = (3√3 / 2)(2 km)²

A = (3√3 / 2)(4)

A ≈ 10.392 km²

Result: The idealized area of one cell is approximately 10.392 km².

3.2 Total Geometric Area of a 19-Cell Planning Cluster

Formula:

A₁₉ = 19 × A

Calculation:

A₁₉ = 19 × 10.3923

A₁₉ ≈ 197.454 km²

Result: The 19-cell cluster has a theoretical summed area of approximately 197.454 km², assuming ideal hexagons without gaps or overlaps.

3.3 Frequency Reuse Fraction and Reuse Distance

Given:

Reuse cluster size, N = 7
Cell radius, r = 2 km

Frequency Reuse Fraction

Formula:

Reuse fraction = 1 / N

Calculation:

= 1 / 7

= 0.1429

Therefore, reuse fraction ≈ 14.29%.

Reuse Distance

Formula:

D = r√(3N)

Substitution:

D = 2√(3 × 7) km

D = 2√21 km

D ≈ 9.165 km

Result: The frequency reuse distance is approximately 9.165 km.

Interpretation

The reuse fraction represents the fraction of the total available channel set assigned to one cell in a simple N-cell reuse pattern. A larger reuse distance can reduce co-channel interference, but can also reduce the number of channels available per cell for a fixed spectrum allocation.

3.4 Assumptions and Capacity-Expansion Options

Hexagonal Approximation

Cells are modeled as regular hexagons with equal radius and idealized boundaries. Real radio coverage depends on terrain, buildings, antenna patterns, transmit power, clutter, weather at higher frequencies and interference.

Geometric coverage is not guaranteed service coverage. A location inside the drawn hexagon may still have weak signal or poor quality; measurements and radio planning are needed.

Cell Splitting

Cell splitting divides a high-demand cell into smaller cells. It can increase spatial reuse and capacity but requires more sites and may increase handoff frequency and interference-coordination needs.

Sectoring

Sectoring uses directional antennas to divide a site into sectors. It can reduce interference from some directions and improve resource reuse, but adds antenna and planning complexity.

Microcells and Picocells

These smaller cells can be added in streets, campuses or busy indoor/outdoor locations to target traffic. They can improve local capacity but increase handoffs and site/backhaul requirements.

Femtocells

Femtocells are small cells, often installed at user premises, for localized indoor coverage. They can improve indoor service but need interference and access coordination.

Recommendation

Start with targeted sectoring and add microcells/picocells in verified high-traffic locations rather than splitting every cell immediately. This stages capital cost and adds capacity where needed. Reassess handoff rates and inter-cell interference after deployment.

4. STAGE 3 – TRAFFIC AND CHANNEL CAPACITY (15 MARKS)

4.1 Offered Busy-Hour Traffic

Given:

Number of subscribers = 1,000
Traffic per subscriber = 0.08 Erlang

Formula:

Offered traffic A = Number of subscribers × Traffic per subscriber

Calculation:

A = 1,000 × 0.08

A = 80 Erlangs

Result: The planning area offers an average of 80 Erlangs of traffic during the busy hour under the supplied per-subscriber traffic assumption.

4.2 Erlang B Blocking Probability

Erlang B models a loss system with C channels, random independent call arrivals, a suitable holding-time model, and no waiting queue.

The recursive calculation is:

B(0,A) = 1

B(c,A) = [A × B(c−1,A)] / [c + A × B(c−1,A)]

For A = 80 Erlangs and C = 100 channels, the recursive Erlang B result is:

B(100,80) ≈ 0.003992

Blocking probability = 0.399%

The result is below the 2% target, so the initial 100-channel group meets the target under the model assumptions.

4.3 Minimum Channels for a Blocking Target of 2%

Testing the channel count using Erlang B:

For 91 channels:

B(91,80) ≈ 2.254%

This fails the target.

For 92 channels:

B(92,80) ≈ 1.922%

This meets the target.

Therefore, the smallest channel count meeting the target is 92 channels.

Estimated 100-channel groups:

Number of groups = ceiling(92 / 100)

= 1 group

This is a first-order channel-group estimate, not a complete site plan. Actual planning must consider geographic traffic distribution, sectorization, frequency reuse, control-channel overhead, busy-hour variation, handoff traffic, spectrum availability, resilience and backhaul.

Trade-Off

More channels reduce blocking but may require additional spectrum or infrastructure. Adding sites can improve capacity and coverage but increases capital cost, energy use, backhaul requirements and coordination complexity.

The offered-traffic model assumes each subscriber contributes an average 0.08 Erlang during the busy hour and that traffic is pooled as represented by the Erlang B model.

5. STAGE 4 – MOBILITY AND HANDOFF ANALYSIS (15 MARKS)

5.1 Distance Travelled During Handoff Overlap

Given:

- Vehicle speed = 60 km/h

- Handoff overlap dwell time = 6 seconds

- Route cell length = 1 km

- Handoff interruption time = 0.5 seconds

- Target cell unavailable probability = 1%

Convert speed from km/h to m/s:

Speed = 60 × 1000 / 3600

Speed = 16.667 m/s

Distance travelled during the 6-second overlap:

Formula:

Distance = Speed × Time

Distance = 16.667 × 6

Distance = 100 metres

Therefore, the vehicle travels approximately 100 metres during the handoff overlap period.

5.2 Estimated Handoffs per Minute

Given route cell length = 1 km and speed = 60 km/h.

Handoffs per hour ≈ Speed / Cell length

= 60 / 1

= 60 handoffs per hour

Handoffs per minute = 60 / 60

Estimated handoffs = 1 per minute

This is an idealized estimate assuming the vehicle crosses one cell boundary per kilometre. Actual handoff frequency depends on cell geometry, route direction, coverage overlap and network configuration.

5.3 Comparison of Handoff Methods

| Handoff Type | Description | Advantages | Limitations |
| --- | --- | --- | --- |
| Hard handoff | The existing connection is released before the new connection is established. | Efficient use of radio resources and commonly used in LTE/NR mobility. | A poorly timed transition may cause a short interruption or call drop. |
| Soft handoff | A mobile can maintain links with multiple base stations during the transition. | Can improve continuity in systems that support it. | Requires additional radio resources and coordination. |
| Softer handoff | The mobile is transferred between sectors controlled by the same base station. | Sector-level coordination may help maintain continuity. | Depends on system architecture and supported radio technology. |

Interpretation

Handoff design aims to maintain service while a user moves between cells. The appropriate method depends on the radio technology and its supported mobility procedures.

5.4 Simplified Call-Drop Risk

The assignment gives a target-cell-unavailable probability of 1%.

Under the simplified assumption that a handoff fails whenever the target cell is unavailable, the probability of failure for one handoff is:

P(failure per handoff) = 1% = 0.01

Therefore, the simplified failure probability per handoff is 1%.

This does not mean that every failed handoff necessarily results in a dropped call. Actual call-drop probability also depends on radio conditions, timing, neighboring cells, handoff procedures, retry mechanisms and network implementation.

The interruption-to-overlap ratio is:

Ratio = 0.5 / 6

= 0.0833

= 8.33%

This ratio compares the interruption duration with the overlap duration. It is not itself a call-drop probability.

5.5 Mobility Recommendations

- Optimize handoff thresholds and timing using drive-test and network measurements.

- Ensure adequate overlap between neighboring cells where practical.

- Monitor handoff failures, call drops and interruption time.

- Improve coverage in weak-signal locations along important transport routes.

- Review target-cell capacity and availability during busy hours.

- Validate the simplified assumptions using real network performance data.

Stage 4 Conclusion

At 60 km/h, the vehicle travels approximately 100 metres during a six-second overlap. With a nominal one-kilometre route cell length, the estimated handoff rate is one per minute. Proper mobility tuning and target-cell availability are important to maintaining service continuity.

6. STAGE 5 – INTERFERENCE AND SPECTRUM MANAGEMENT (20 MARKS)

6.1 Co-Channel Carrier-to-Interference Ratio

Given:

Desired signal power, S = −70 dBm

Co-channel interference, I₁ = −85 dBm

Adjacent-channel interference, I₂ = −90 dBm

Target SINR = 15 dB

The co-channel-only carrier-to-interference ratio is:

C/I = S − I₁

C/I = −70 − (−85)

C/I = 15 dB

The co-channel-only C/I meets the 15 dB numerical target. However, this calculation excludes adjacent-channel interference and thermal noise.

6.2 Combining Interference Powers

Decibel values cannot be directly added. Convert the interference powers to linear units first.

For power in dBm:

P(mW) = 10^(P(dBm)/10)

For the co-channel interferer:

I₁ = 10^(−85/10) mW

I₁ ≈ 0.000003162 mW

For the adjacent-channel interferer:

I₂ = 10^(−90/10) mW

I₂ = 0.000001 mW

Total interference:

I_total = I₁ + I₂

I_total ≈ 0.000004162 mW

Convert the result back to dBm:

I_total(dBm) = 10 log₁₀(I_total in mW)

I_total ≈ −83.807 dBm

6.3 SINR Using the Supplied Interference Values

The assignment does not provide a separate thermal-noise value. Therefore, for this calculation, noise is neglected and the result is an interference-limited approximation.

SINR ≈ S / I_total

In decibel form:

SINR(dB) = S(dBm) − I_total(dBm)

SINR = −70 − (−83.807)

SINR ≈ 13.807 dB

Comparison with the Target

Target SINR = 15 dB

Calculated SINR ≈ 13.807 dB

Difference = 15 − 13.807

= 1.193 dB below the target

Therefore, the co-channel-only C/I reaches 15 dB, but the combined-interference result is approximately 13.807 dB. Under the stated noise-neglecting assumption, it does not meet the 15 dB SINR target.

6.4 Interference-Mitigation Methods

Power Control

Adjusting transmit power can reduce unnecessary interference while maintaining an adequate desired signal. Power must be optimized carefully to avoid weakening the desired link.

Antenna Optimization

Directional antennas, suitable antenna tilt and beamforming can reduce energy directed toward unintended receivers and interfering cells.

Frequency Planning

Appropriate frequency allocation and reuse planning can reduce co-channel interference. Guard bands and suitable filtering can help limit adjacent-channel interference.

Sectoring

Sector antennas can improve spatial separation and reduce interference arriving from certain directions, depending on the deployment.

Interference Coordination

Neighboring cells can coordinate radio-resource use and scheduling to reduce harmful interference, particularly at cell edges.

Measurement and Optimization

Operators should measure SINR, reference-signal quality, throughput and interference across the service area. The mitigation method should be selected based on the measured cause of poor performance.

6.5 Wavelength and Free-Space Path Loss at Different Frequencies

Given:

Distance, d = 100 m

Speed of light, c = 3 × 10⁸ m/s

Wavelength formula:

λ = c / f

Free-space path-loss formula:

FSPL(dB) = 20 log₁₀(4πd / λ)

The calculation assumes ideal free-space propagation and does not include walls, terrain, atmospheric effects or antenna gains.

Case 1: 900 MHz

Frequency = 900 × 10⁶ Hz

Wavelength:

λ = (3 × 10⁸) / (900 × 10⁶)

λ ≈ 0.3333 m

FSPL:

FSPL = 20 log₁₀(4π × 100 / 0.3333)

FSPL ≈ 71.53 dB

Case 2: 3.5 GHz

Frequency = 3.5 × 10⁹ Hz

Wavelength:

λ = (3 × 10⁸) / (3.5 × 10⁹)

λ ≈ 0.08571 m

FSPL:

FSPL = 20 log₁₀(4π × 100 / 0.08571)

FSPL ≈ 83.32 dB

Difference compared with 900 MHz:

83.32 − 71.53 = 11.79 dB approximately

Case 3: 28 GHz

Frequency = 28 × 10⁹ Hz

Wavelength:

λ = (3 × 10⁸) / (28 × 10⁹)

λ ≈ 0.01071 m

FSPL:

FSPL = 20 log₁₀(4π × 100 / 0.01071)

FSPL ≈ 101.38 dB

Difference compared with 900 MHz:

101.38 − 71.53 = 29.85 dB approximately

Summary Table

| Frequency | Wavelength | Free-Space Path Loss at 100 m |
| --- | --- | --- |
| 900 MHz | 0.3333 m | 71.53 dB |
| 3.5 GHz | 0.08571 m | 83.32 dB |
| 28 GHz | 0.01071 m | 101.38 dB |

Interpretation

For the same distance in ideal free space, the calculated path loss increases with frequency when the comparison uses the same reference assumptions.

Lower frequencies are generally useful for broader coverage, while mid-band spectrum offers a balance between coverage and capacity. Higher-frequency spectrum can provide substantial bandwidth but often requires denser infrastructure and careful planning because practical coverage can be affected by blockage and penetration loss.

6.6 Stage 5 Recommendations

- Investigate the combined-interference SINR shortfall before increasing transmit power.

- Improve antenna direction, tilt and beamforming where measurements support these changes.

- Review frequency assignments and adjacent-channel filtering.

- Apply inter-cell interference coordination where appropriate.

- Use 900 MHz for broad coverage where spectrum and technology availability permit.

- Use 3.5 GHz to increase capacity in locations with demonstrated traffic demand.

- Use 28 GHz selectively for dense hotspots and other suitable locations.

- Validate predicted coverage and performance using field measurements.

Stage 5 Conclusion

The co-channel-only C/I is 15 dB, but combining the supplied interference powers gives an approximate interference-limited SINR of 13.807 dB. This is 1.193 dB below the target. The propagation comparison also shows increasing free-space path loss at 3.5 GHz and 28 GHz relative to 900 MHz at the same distance.

7. STAGE 6 – SPECTRUM COORDINATION AND DEPLOYMENT RECOMMENDATION (20 MARKS)

7.1 Role of International Spectrum Coordination

International Telecommunication Union – Radiocommunication Sector (ITU-R)

ITU-R develops international recommendations and technical studies for radiocommunication systems. Its work supports compatible use of radio frequencies and the development of technical standards and procedures.

World Radiocommunication Conference (WRC)

The WRC reviews and, when agreed by participating administrations, revises the Radio Regulations. These decisions help coordinate international spectrum use and address interference between radio services.

National Regulatory Bodies (BR)

National regulatory bodies administer spectrum use within their jurisdictions, following applicable national law and international obligations. Their responsibilities may include licensing, assigning frequencies, enforcing technical conditions and monitoring interference.

The exact powers and procedures of a regulator depend on the country and its legal framework.

7.2 Proposed Phased Deployment Plan

Phase 1: Establish Coverage and Baseline Performance

- Assess existing coverage, subscriber distribution and busy-hour traffic.

- Identify weak-signal areas, congested cells and important transport routes.

- Use lower-frequency spectrum where available and appropriate for broad-area coverage.

- Measure handoff failures, dropped calls and interference.

Purpose: Establish reliable baseline coverage and identify the highest-priority service gaps.

Phase 2: Expand Capacity in High-Demand Areas

- Introduce or optimize mid-band capacity, such as 3.5 GHz where licensed and available.

- Consider sectoring or additional small cells in high-traffic areas.

- Review backhaul capacity, site availability and power requirements.

- Validate frequency reuse and interference performance.

Purpose: Increase capacity where traffic measurements show that existing resources are insufficient.

Phase 3: Targeted High-Frequency Deployment

- Consider 28 GHz in dense hotspots where substantial bandwidth is useful.

- Assess blockage, indoor penetration, line-of-sight availability and site density.

- Use suitable beamforming and carefully planned site locations.

- Verify performance under realistic traffic and mobility conditions.

Purpose: Add localized high capacity without relying on high-frequency coverage for every location.

Phase 4: Continuous Optimization

- Monitor traffic growth and geographic demand.

- Review blocking probability, throughput, SINR and handoff performance.

- Adjust power, antenna configuration and frequency allocation.

- Reassess expansion priorities as usage patterns change.

Purpose: Keep the network aligned with changing demand while controlling cost and interference.

7.3 Technical and Operational Uncertainties

The proposed plan depends on several factors:

- Spectrum availability: The required bands may not be licensed or available in every region.

- Propagation: Real-world coverage differs from ideal free-space calculations because of buildings, terrain and blockage.

- Traffic demand: Subscriber counts and traffic per subscriber may change over time.

- Site and backhaul availability: New capacity requires suitable locations, power and backhaul.

- Interference: Neighboring networks and cells may affect the achievable SINR.

- Device compatibility: Not every device supports every frequency band or radio feature.

- Regulatory conditions: National licensing requirements and technical limits must be followed.

- Cost and energy: Denser deployments can increase capital, operational and energy costs.

These uncertainties should be handled through staged investment, measurements and periodic reassessment rather than assuming the theoretical calculations fully predict field performance.

7.4 Integrated Engineering Recommendation

The operator should adopt a coverage-first, capacity-led deployment strategy.

First, establish broad and reliable coverage using suitable lower-frequency spectrum. Second, expand mid-band capacity in locations where measured traffic demand justifies investment. Third, introduce high-frequency capacity selectively in dense hotspots where the bandwidth benefit justifies the additional site and coverage requirements.

At every stage, the operator should monitor interference, blocking probability, handoff failures and user performance. Spectrum coordination and compliance with national regulations should be treated as continuing requirements throughout deployment.

8. OVERALL CONCLUSION

This report examined the evolution of mobile communications and the engineering trade-offs involved in cellular coverage, channel capacity, mobility, interference and spectrum planning.

At 800 MHz, the wavelength is 0.375 m. Under the idealized 10 MHz and 30 kHz assumption, 333 complete channels fit before accounting for practical channel-plan constraints.

For a cell radius of 2 km, the idealized hexagonal area is approximately 10.392 km². A 19-cell cluster has a summed geometric area of approximately 197.454 km². With a reuse cluster size of seven, the reuse fraction is 14.29% and the reuse distance is approximately 9.165 km.

The offered busy-hour traffic is 80 Erlangs. Under the Erlang B model, 100 channels produce approximately 0.399% blocking, while 92 channels are the minimum to achieve blocking below 2%.

At 60 km/h, the vehicle travels about 100 metres during a six-second handoff overlap. Under the simplified one-kilometre cell-length assumption, the estimated handoff rate is one per minute.

For the supplied interference powers, the co-channel-only C/I is 15 dB. After combining the co-channel and adjacent-channel interference powers, the interference-limited SINR is approximately 13.807 dB, below the target of 15 dB.

The free-space path-loss calculations at 100 m are approximately 71.53 dB at 900 MHz, 83.32 dB at 3.5 GHz and 101.38 dB at 28 GHz.

Overall, the recommended deployment uses lower-frequency spectrum for broad coverage, mid-band spectrum for additional capacity, and high-frequency spectrum for selected dense hotspots. Field measurements, regulatory coordination and phased investment are essential to validate the design and manage uncertainty.

9. REFERENCES

- International Telecommunication Union (ITU), Radiocommunication Sector (ITU-R), official publications and recommendations.

- International Telecommunication Union (ITU), Radio Regulations and World Radiocommunication Conference information.

- GSM Association (GSMA), technical and educational materials on mobile communication systems.

- 3rd Generation Partnership Project (3GPP), specifications and technical information for GSM, UMTS, LTE and 5G NR.

- Assignment brief: ECA56 – Advanced Wireless Communication Systems, CO1 Integrated Engineering Analysis and Planning Assignment.

Note: Add the exact editions, publication years, document titles and URLs of sources actually consulted before submission.